FPGA-Based High-Performance Bilinear Image Scaler

Team Name RONIT
Team Members:- Rohanraj Mallick(24095094)- sophomore
                                 Nitin Kumar Rai(24095072)- sophomore

Event: I-Chip ’26 / UDYAM ’26

1. Abstract

This project presents a hardware-efficient implementation of a Bilinear Interpolation Image Scaler designed for FPGA deployment. The core objective was to upscale a 100 \times 100 input image to a 400 \times 400 resolution while maintaining high visual fidelity and maximizing throughput. By utilizing a 5-stage pipelined architecture and Q8.8 fixed-point arithmetic, the design achieves a throughput of 1 pixel per clock cycle without the need for floating-point units or hardware dividers. Verification against a Python-based Golden Model confirms a Structural Similarity Index (SSIM) of 1.0000 and a Peak Signal-to-Noise Ratio (PSNR) of 49.03 dB, demonstrating near-perfect reconstruction accuracy suitable for real-time display controllers and pre-processing in computer vision pipelines.

2. Introduction

Image scaling is a fundamental operation in digital signal processing, required for everything from resizing videos on a monitor to preparing datasets for Convolutional Neural Networks (CNNs). While software implementations are flexible, they are often too slow for real-time applications. Hardware acceleration on FPGAs offers massive parallelism but requires careful resource management.

The "ScaleX" problem statement challenged us to implement high-quality scaling (Bilinear Interpolation) rather than simple Nearest Neighbour, which produces blocky artifacts. Our solution focuses on three key design metrics:

- Throughput: Processing one full RGB pixel every clock cycle.

- Accuracy: Matching the mathematical precision of software models.

- Efficiency: Eliminating costly division logic in favour of bit-shift operations.

3. Mathematical Formulation

Bilinear interpolation estimates a new pixel value by taking a weighted average of the four nearest neighbors in the source image.

The intensity I at target coordinate (x, y) is given by:

I(x,y) = (1-a)(1-b)P_{00} + a(1-b)P_{10} + (1-a)bP_{01} + abP_{11}

Where a and b are the fractional distances from the top-left neighbour.

3.1 Fixed-Point Optimization (Q8.8 Format)

To avoid floating-point hardware, we mapped the input domain to the output domain using a scaling factor derived at compile time:

Scale Factor = {Input Width \times 256}{Output Width}

Instead of standard division, the hardware performs integer multiplication followed by a logical right shift. The upper 16 bits of the result represent the Integer Coordinate (memory address), while the lower 8 bits represent the Fractional Weight (a or b). This allows the address generation logic to run in a single clock cycle.

4. Hardware Architecture

The design avoids the bottlenecks of standard Finite State Machines (FSMs) by implementing a fully pipelined datapath.

4.1 The 5-Stage Pipeline

Data flows continuously through the system, separated by registers to maximize frequency (F_{max}).

- Stage 1 (Counter Generation): Iterates through output coordinates (0,0) to (399,399).

- Stage 2 (Coordinate Mapping): Applies the fixed-point scaling factor to determine source coordinates and fractional weights. Boundary logic clamps coordinates to prevent memory overflow.

- Stage 3 (Memory Fetch & Pre-Compute): Reads the 4 neighbor pixels from memory. Simultaneously, the four area weights (W_{00}, W_{10}, W_{01}, W_{11}) are pre-calculated to reduce critical path delay in the next stage.

- Stage 4 (Parallel DSP Core): Three parallel math engines process the Red, Green, and Blue channels simultaneously using the pre-calculated weights. This utilizes the FPGA’s DSP48 slices efficiently.

- Stage 5 (Write-Back): The resulting 8-bit channels are packed into a 24-bit word and written to the output memory.

4.2 Memory Organization

To support the 1 pixel/cycle throughput, the memory architecture was widened from standard 8-bit to 24-bit (Packed RGB).

- Input: mem_in holds packed \{R, G, B\} data.

- Output: mem_out stores the processed pixels.

This eliminates the need to cycle through memory three times for a single pixel, effectively tripling the bandwidth compared to a serial approach.

5. Verification & Results

The design was verified using a Python full_scaler model(Reference) to ensure bit-accurate performance.

5.1 Methodology

- Input Generation: A Python script converted a standard PNG image into a 24-bit Hexadecimal memory file (image_in_24b.hex).

- Simulation: The Verilog design was simulated in Xilinx Vivado.

- Output Analysis: The simulation output (image_out_24b.hex) was compared against the Python Golden Model using scikit-image.

5.2 Quantitative Metrics

| Metric | Value | Significance |
| --- | --- | --- |
| MSE (Mean Squared Error) | 0.8128 | Average pixel error is < 1.0 (out of 255). |
| PSNR (Peak Signal-to-Noise Ratio) | 49.03 dB | Indicates extremely high fidelity (Values >40dB are considered excellent). |
| SSIM (Structural Similarity Index) | 1.0000 | Perfect structural match to the theoretical model. |

5.3 Visual Comparison

The visual output confirms the metrics, with smooth gradients and no visible aliasing or "zipper" artifacts often seen in poorer implementations.

6. Conclusion & Future Work

The proposed pipelined architecture successfully meets all design constraints of the ScaleX challenge. By leveraging parallel processing and fixed-point arithmetic, we achieved a throughput of 160 megapixels per second (at 160MHz) with minimal resource usage.

Future iterations of this design could implement Line Buffers (FIFOs) instead of full-frame memory. This would allow the scaler to process infinite video streams (e.g., HDMI input) without being limited by the FPGA's on-chip Block RAM capacity.